LEAF2 Instruction Sheet

The LEAF2 model is based on the methodology of the Lifetime Evaluation of Alternative Fuels (LEAF) model developed by McCallum, the details of which are published in the Report of the Alternative Fuels Group of the Cleaner Vehicle Task Force (DTI 2000). To acknowledge its origins and to signify that several extensions and modifications have been made to the LEAF approach, the model used for this thesis is known as ‘LEAF2’.

Figure 1 : Structure of LEAF2 Model

Assumptions

Car/Bus Charts

Car/Bus Base Data

Car/Bus

Hart Analysis

Hydrogen cost curves

Fuel Data

LEAF2 Overview

The LEAF2 model estimates the costs and emissions for cleaner and conventional vehicle technologies introduced over the period 2002-2012. Two vehicle types are considered; cars and buses. The model uses vehicle and fuel input data based on figures from published sources (refer to Chapter 4 of thesis). The model first calculates annual lifecycle emissions and costs associated with each vehicle type and technology option. These are then summed over the life of the vehicle giving the total lifetime emissions and costs. (Chapter 3 of the thesis provides a full account of the methodology used.)

The model generates two environmental measures. These are ‘lifetime NOx’ and ‘lifetime greenhouse gas’ (GHG) emissions. Three economic measures are also produced. The first of these is the ‘lifetime private cost’, which is a measure of the economic impact from the perspective of the user. The second is the ‘lifetime public cost’, which is a measure of the economic impact from the perspective of the UK as a whole. The third economic measure is the ‘(public) cost-effectiveness’, which is the emissions reduction per public pound spent.

For each vehicle type, comparisons are made with one of three baselines. The baselines include conventional vehicles and those with electric drive-trains:

- Petrol/diesel ICE vehicles (ICEs) – main baseline.

- Petrol/diesel ICE hybrid electric vehicles (HEVs).

- Battery electric vehicles (BEVs) (Year 2002-2012 generating mix).

Lifecycle analysis of emissions

As greenhouse gases (GHGs) emitted at all locations contribute to global warming, greenhouse gas emissions are calculated on a lifecycle basis and include fuel production, processing, distribution and use. The three main GHG emissions associated with road transport are carbon dioxide, nitrous oxide and methane. Therefore, where possible, production and vehicle emissions of these three gases are accounted for within the analysis presented. Note that these emissions are combined to provide a single measure of greenhouse gas emissions by using accepted global warming potentials (GWP) for each of the gases. These provide a weighting for each gas, which is a measure of their effect on atmospheric warming over a determined time period. As air quality impacts are focused at the local level, regulated emissions (CO, NOx, HCs, PMs) are normally analysed on a vehicle emission basis. However, as many of the technology options that are considered by the model involve fuel production at the local level (e.g. on-site at fuel stations), all regulated emissions are also conducted on a lifecycle basis.

Sustainable emissions targets

The LEAF2 model uses emissions sustainability targets. As introduced in Chapter 1 of the thesis, these set eco-capacity limits for NOx and GHGs, which represent a 75% reduction in lifecycle emissions. These are based on Euro 4 (NOx) and Euro 2 (GHGs) operation. There is no NOx target for buses as Euro 4 regulated emissions are specified in gms/kWh engine output while the model makes estimates on a per km basis.

Lifetime analysis of costs

The LEAF2 model uses two definitions of cost. These are lifetime private costs and lifetime public costs. Private costs are the costs faced by the vehicle user (individual or bus operator). The user pays the cost of the vehicle, fuel, maintenance, and all taxes incurred. Private cost is useful as an indicator of market response. Combined with a behavioural model (not included in LEAF2), the relative private cost can be used to determine the up-take of different vehicle types.

Public costs are the costs to society as a whole (or 'UK plc'). Public costs can be equated with resource costs because they reflect land, labour and capital investment. Taxes are not included, because tax paid by individuals remains within the economy. Public cost is therefore a useful measure with which to inform policy and is so used by government.

Cost-effectiveness

The cost-effectiveness is found by calculating the ratio of emissions reduced to public pounds spent. Public cost is used for this quotient as environmental savings are considered as a 'social' benefit. Cost-effectiveness is expressed as a single figure for each pollutant over the lifetime of the vehicle and is defined relative to a baseline. Cost-effectiveness is useful in comparing technology options. A high positive cost-effectiveness is preferable, meaning that a larger reduction in emission for the same investment. However, a negative value is interpreted as reduced emissions at less cost. It is important to note that some fuels/technologies may be associated with a high cost-effectiveness though the magnitude of emissions reduction may be low.

Instructions for use

- Load CD ROM

- 2 folders each containing 7 Excel Files - Assumptions, Hydrogen cost curves, Hart analysis, Fuel Data, Car/Bus Base Data, Car/Bus, Car/Bus Charts.

- Select 'LEAF2 Car' or 'LEAF2 Bus' folder - and Open

- Select ALL files - and Open (accept all link updates and enable all micros)

- Select 'Assumptions' spreadsheet (use Window pull-down menu)

- Enter global parameters - only enter in YELLOW boxes.

- Choose comparator year - 2002, 2007, 2012

- Choose vehicle use - see options

- Choose PowerShift capital subsidy - Auto-select chooses 75% (2002), 50% (2007), 0% (2012)

- Choose fuel duty option (2012) - for hydrogen, OFF (0), LOW (75p/kg), HIGH (180p/kg car, 230p/kg bus)

- Select 'Car/Bus Charts' (use Window pull-down menu)

- View/Print charts - Ch4 or Ch5 spreadsheets - SEE KEY BELOW

Chapter 4 (Ch4) displays:

Interpreting Charts

Private costs - values above baseline indicate low market up-take.

Public costs - values above baseline indicate increased societal costs.

NOx Emissions - compare with sustainable target for 2012 case.

GHG Emissions - compare with sustainable target for 2012 case.

Cost-effectiveness - the higher the value, the larger the emissions

benefit per public pound. Negative values indicate emissions

benefit at LOWER cost (the ultimate option).

- Lifetime Private Cost

- Lifetime Public Cost

- Lifetime NOx Emissions

- Lifetime GHG Emissions

- Chapter 5 (Ch5) displays:

- Lifetime Private Cost

- Lifetime Public Cost

- NOx Reduced per Public Pound

- GHGs Reduced per Public Pound

- Return to 'Assumptions' spreadsheet - repeat from Step 5.

KEY FOR CHARTS

| Symbol | Off-board fuel | Off-board fuel processing | On-board fuel processing | On-board fuel | Main power system |
| --- | --- | --- | --- | --- | --- |
| pet/die --- ice | Petrol/Diesel | None | None | Petrol/Diesel | Internal combustion engine |
| pet/die --- hev | Petrol/Diesel | None | None | Petrol/Diesel | Hybrid electric engine |
| grd --- bev | Electric grid | None | None | Electricity | Battery - electric motor |
| nat gen hfc | Natural gas | Reformer | None | Hydrogen | H2 fuel cell – electric motor |
| ren gen hfc | Renew. Elec. | Electrolysis | None | Hydrogen | H2 fuel cell – electric motor |
| grd gen hfc | Electric grid | Electrolysis | None | Hydrogen | H2 fuel cell – electric motor |
| pet ref hfc | Petrol | None | Reformer | Hydrogen | H2 fuel cell – electric motor |
| mth ref hfc | Methanol | None | Reformer | Hydrogen | H2 fuel cell – electric motor |
| lpg ref hfc | LP gas | None | Reformer | Hydrogen | H2 fuel cell – electric motor |
| nat ref hfc | Natural gas | None | Reformer | Hydrogen | H2 fuel cell – electric motor |

DTI (2000) The Report of the Alternative Fuels Group of the Cleaner vehicle task Force Report, Department of Trade and Industry, Automotive Directorate, The Stationery Office, London.

The GWP for carbon dioxide is taken as unity; the GWP for methane is equal to 21 for a 100 year time horizon (DTI 2000). Nitrous oxide has not been assessed in this thesis due to lack of available production and vehicle data.