Appendix 2A Additional material not included in thesis

Table 1.1a Sands and chemically modified sands and their applications for water treatments

| # Sand filters | # Application | # Removal mechanisms | # Suitable for | # Unsuitable for | # Reference |
| --- | --- | --- | --- | --- | --- |
| # Anthracite | # Raw water treatment | # Filtration adsorption | # Particles | # Metals | # Merkle et al., 1996 |
| # Anthracite, MnO2 coated sand | # Drinking water treatment | # Filtration adsorption | # Mn2+ and particles | # Other metals | # Merkle et al., 1997 |
| # Dual media sand filters | # Water purification | # Filtration adsorption | # Turbidity, bacteria, OM | # Metals | Burdurlu and Aydin 1999 |
| # Anthracite sand | # Secondary treatment | # Filtration | # Giardia cysts Crytosporidim | # Turbidity metals | Swerfteger et al., 1999 |
| # Combined flocculator and sand filter | # Secondary sewage effluent | # Filtration flocculation | # Coliform streptococi | # Metals | Peiris et al., 1998 |
| # Floating medium sand filtration | # Prawn farm effluent | # Filtration flocculation | # Particles | # Metals, bacteria | Ngo and Vigneswaran 1998 |
| # Iron oxide coated sand | # Water treatment | # Adsorption filtration | # Cu, Cd, Pb, Ni, Zn, SeO3 AsO3 | # Other metals | # Benjamin et al., 1996 |
| # Iron oxide coated sand | # Water purification | # Filtration adsorption | # Particles, heavy metals | # Other metals | # Lo et al., 1997 |
| # Iron oxide coated sand | # Low cost Home water filter | # Physio-chemical | # Turbidity bacteria and viruses | # Metals | Ahammed and Chadhuri 1999 |
| # Intermittent sand filters | # Primary effluent treatment | # Adsorption Filtration | # N, P, COD, BOD | # Metals | Van Buuren et al., 1999 |

Table 1.1a contd

| # Sand filters | # Application | # Removal Mechanisms | # Suitable for | # Unsuitable for | # Reference |
| --- | --- | --- | --- | --- | --- |
| # Iron oxide treated Ottawa sand | # Water purification | # Physical capture of microbes | # E-coli Streptococus Salmonella | # Metals | Truesdail et al., 1998 |
| # Iron oxide and Al (OH)3 | # Micro-organisms from de-chlorinated tap water | # Physical and chemical | # e-coli, poliovirus, cholerae coliphage | # Metals | Lukasik et al., 1999 |
| # Iron oxide coated sand | # Water purification | # Chemical sorption | # Metals and humic acids | # Alkali and alkaline metals | Lai et al., 2001, 2002 |
| # MnO2 coated quartz sand | # Metal removal | # Oxidation precipitation | # As(III) As(V) | # Particles other metals | # Borho and Wilderer 1996 |
| # Manganese green sand | # Drinking water treatment | # Filtration oxidation | # As(III) | # Other metals | Viraraghavan et al., 1999 |
| # MnO2 coated sand | # Primary effluent treatment | # Chemisorbed | # Heavy metals | # Bacteria | Lo et al., 2002 |
| # New sand Vs used sand | # Groundwater treatment | # Adsorption Filtration | # Fe2+ | # Other metals | Sharma et al., 1999 |
| # Quartz sand | # Chlorinated effluent | # Physical and chemical | # Bacteriophage Particulates | # Metals | Redman et al., 1999 |
| # Rapid sand filters | # Primary water treatment | # Filtration adsorption | # Plankton Bacteria | # Metals | Clasen, 1998 |
| # Rapid and slow sand filters | # Surface water purification | # Coagulation flocculation and adsorption | # Radio-nuclides | # Caesuim | Gafvert et al., 2002 |

Table 1.1a contd

| # Sand filters | # Application | # Removal Mechanism | # Suitable for | # Unsuitable for | # Reference |
| --- | --- | --- | --- | --- | --- |
| # Sand various sizes | # Sedimentation effluent | # Physical filtration | # Particles | # Metals, bacteria | # Kau and Lawler 1995 |
| # Sand-biologically activated carbon | # Water purification | # Filtration adsorption | # Humics organo-chlorines | # Metals bacteria | # Thibault et al., 1997 |
| # Sand inoculated with bacteria | # Polishing stage | # Physical and chemical | # Heavy metals | # For Water <18 ºC | # Deils et al., 2003 |
| # Sand and Granular activated carbon | # Water purification | # Filtration adsorption | # Organic carbon, tri-halomethanes | # Metals | # Collins et al., 1996 |
| # South African Silica sand | # Water treatment | # Physio-chemical | # Particles | # Metals | # Ceroni et al., 1997 |
| # Slow sand filtration (SSF) | # Water purification | # Bio-accumulation | # Heavy metals Bio-films | # Particles, other metals | Muhammad et al., 1998 |
| # SSFs with natural fabrics | # Water purification | # Filtration | # Particulate matter | # Metals | Luxton & Graham 1998 |
| # SSF | # Water purification | # Physical and chemical | # Virus reduction | # Metals | # Reisenberg et al., 1995 |
| # SSF | # Potable water treatment | # Physical and chemical | # Pesticides, bacteria, virus | # DOC, turbidity | # Lambert and Graham 1995 |

Gafvert, T., Ellmark, C., Holm, E. (2002) Removal of radio-nuclides at a waterworks. Journal of Environmental Radioactivity, Vol. 63, 105-115

L, Diels., P. H.,  Spaans, S. V.  R.,  Hooyberghs,  L., Ryngaert,  A., Wouters, H., Walter,  E.,  Winters, J., Macaskie, L., Finlay, J., Pernfuss, B., Woebking, H.,  Pu¨mpel, T.,  Tsezos, M. (2003) Heavy metals removal by sand filters inoculated with metal sorbing and precipitating bacteria. Hydrometallurgy, Vol. 71, 235-241

Lai, C.H., Lo, S.L., Lin, C.F. 1994, Evaluating an Iron Coated Sand for removing Copper from water.  Water Science Technology, 30. (9) 175 – 182 R

Lai, C.H., Chen, C.Y., ShiH, P.H. and Hsia, T.H. (2000), Competitive adsorption of copper and lead ions on an iron-coated sand from water. Water Science & Technology, Vol. 42, (3-4), 149-154 R

Lai, C.H., Chen, C.Y. (2001). Removal of metal ions and humic acid from water by iron-coated filter media. Chemosphere, Vol. 44, 1177-1184 R

Lai, C.H., Chen, C.Y., Wei, B.L., Yey, S.H. (2002). Cadmium adsorption on geothite-coated sand in the presence of humic acid. Water Research, Vol. 36, 4943-4950 R

Lo, S.L., Jeng, H.T. & Lai, C.H. (1997). Characteristics and adsorption properties of iron coated sand. Water Science and Technology, Vol. 35, (7), 63-70

Muhammad, Nur., Parr, J., Smith, M. D. & Wheatley, A. (1998). Adsorption of heavy metals in slow sand filters.  In the Proceedings of 24th WEDC Conference, Sanitation for all, Islamabad, Pakistan, 346-349 R

Muhammad, Nur., Parr, J., Smith, M. D. & Wheatley, A. (2000). Influences of Organic Material OnThe Performance of Slow Sand Filters in Removing Heavy metals from Surface Water. In: Proceedings World Filtration Congress, 2000. Vol. 1, 363-366 R

Table 1.2a Agricultural and industrial wastes used for the production of activated carbons

| Material | Country | Metal | Reference |
| --- | --- | --- | --- |
| Algal biomass | U.K | Pb2+, | Malik et al., 2002 |
| Almond shells | USA | Cu2+, | Toles et al., 2002 |
| Apricot stones | Turkey | Cu2+, Pb2+ | Beker et al., 2003 |
| Aquatic plants | India | Cu2+, Pb2+ | Shekinah et al., 2002 |
| Blast furnace slag | India | Cu2+, Ni2+ | Gupta, 1998 |
| Bone charcoal | U.K. | Cu2+, Pb2+ | Lewis, 1995 |
| Coal | U.K. | Cr3+, Cr6+ | Laktos et al., 2002 |
| Coffee beans | China | Cu2+, Cd2+, | Minamisawa et al., 2005 |
| Coconut pith | Pakistan | Cd2+, Cr6+, | Kadirvelu and Namasivayam, 2003 |
| Coir pith | India | Hg2+, Cr6+, Ni2+ | Namasivayam and Sangeetha, 2006 |
| Fly ashes | India | Cu2+, Zn2+, | Gupta et al., 1998 |
| Hazelnut shells | Greece | Ni2+ | Demirbas et al., 2002 |
| Lignite | UK | Cd2+, Cu2+, Hg2+ | Allen and Brown, 1995 |
| Moringa oeilfera | Pakistan | Pb2+ | Nadeen et al., 2007 |
| Nut hulls | India | Cd2+, Cu2+, | Madhava-Rao et al., 2006 |
| Olive stones | Italy | Cu2+, Cr6+, Pb2+ | Paganelli et al., 1991 |
| Peanut shells | Portugal | Cd2+, Cu2+, | Volpe et al., 2003 |
| Peat | USA | Cr3+, Cr6+ | Mohan and Pittman, 2006 |
| Petroleum coke | Mexico | Cd2+, Hg2+ , Pb2+ | Ramerirez et al., 2000 |
| Seed husks | Country | Cu2+, Cr6+, | Warhurst et al., 1997 |
| Spent tyres | Brazil | Cd2+, Cr3+, Cu2+, | Sollars et al., 2001 |
| Straw | India | Cd2+, | Kannang and Rengasamy, 2005 |
| Waste ashes | China | Cd2+, Hg2+ | Zhang and Itoh, 2002 |

References not listed in thesis

Minamisawa, S. Nakajima, H., Minamisawa, S.Yoshida, J., Takai, N.  (2005) Removal of Copper(II) and Cadmium(II) from Water Using Roasted Coffee Beans. In: Environmental Chemistry, Pages 259-265, Springer-link, Berlin Heidelberg

Kannang, G., Rengasamy, G. (2005) Comparison of cadmium ion adsorption on various activated carbons. Water, Air and Soil Pollution, Vol. 163 (1-4), 185-201

Warhrst, A. M., McConnachie, G. L., Pollard, S. J. T. (1997) Characterisation and applications of activated carbon produced from Moringa oleifera seed husks by single step pyrolysis. Water Research, Vol. 31 (4), 759-766

Nadeem, M., Mahmood, A., Shahid, S. A., Shah, S. S.,  Khalid, A. A., McKay, G (2007)  Sorption of lead from aqueous solution by chemically modified carbon adsorbents. Journal of Hazardous Materials, (In Press) doi:10.1016/j.jhazmat.2006.05.098

Mohan, D., Pittman Jr., C. U. (2006) Activated carbons and low cost adsorbents for remediation of tri and hexavalent chromium from water. Journal of Hazardous Materials   Vol. B137, 762-811

Namasivayam, C., Sangeetha, D. (2006) Recycling of agricultural solid waste, coir pith: Removal of anions, heavy metals, organics and dyes from water by adsorption onto ZnCl2 activated coir pith carbon. Journal of Hazardous Materials, Vol. B135, 449-452

M. Madhava Rao, A. Ramesh, G. Purna Chandra Rao, K. Seshaiah, (2006) Removal of copper and cadmium from the aqueous solutions by activated carbon derived from Ceiba pentandra hulls. Journal of Hazardous Materials, Vol. B129, 123-129